Model No.:RHBK016

### Cordless Automatic Electric Kettle

Operating Instructions

Please read this operating Instructions before use

Standard No．：

Products:

Thank you for choosing Cordless Automatic Electric Kettle RHBK016！

This product features unique automatic control mode: concealed heating element, high thermal efficiency, safety device, power and time saving. It will not cause pollution of any kind, with a special design meeting health standard for drinkable water Furthermore, the streamline appearance enhances it elegance. The unit is designed for household use only.

Parts identification:

RHBK016:

Feature:

1.  Cordless container, 360°rotation.

2.  Double layers good for warmth and energy saving.

3.  Time saving.

4.  Concealed heating element，easy to clean.

5.  Transparent sight gauge.

6.  Working indicator，automatically cut off power supply after water boiling.

7.  Automatically cut off power supply when water is boiled off. Protection against over-heated.

Know your kettle before first use:

1.  Remove all packing materials from the outside and inside of

kettle (To remove the lid, please see section 'Removing and replacing the lid').

2.  Make sure the voltage marked on the base or on the kettle is corresponds to local voltage before using.

3.  If an extension lead is required it must be earthed.

- Boil the kettle twice to clean the inside before use and discard the boiled water.

- Use thermoplastic body must concert with base.

How to use：

- Push the button to open the lid, pour in water and the close the lid firmly or pour in water directly from the spout .(The assembly &disassembly of lid as the following illustration.)

Caution: Do not fill the kettle with water above the maximum water level mark. If you do, hot water will overflow. Do not fill the kettle with water below the minimum water level mark. If you do, the kettle will not operate normally.

2.  Plug in the base, put the kettle on the base according to the direction of this manual.

3. The indicator lights up, the kettle is working. If the indicator does not light up, please press the power SWITCH(on), the kettle will begin to working,and the indicator lighting up. After the water is boiled, the power supply will be automatically cut off and the indicator will goes off.

- If you want to cut off power before the water is boiled, remove the kettle off the base ,press the power SWITCH(off) or unplug the power cord.

WARNING：

1.  Keep the child away from the kettle when it is boiling.

2.  Never use the kettle without the lid on.

3.  This appliance should not be left unattended during use.

4.  Do not touch the exterior as it gets hot or allow your hand to come in contact with hot steam from the spout or lid, particularly when refilling.

5.  To avoid the kettle overheating , Do not connect the power without water in the kettle.

6.  The unit is for boiling water only. Do not use it to heat other liquids to avoid damage.

7.  Contact your supplier for replacement of damaged power cord to avoid danger.

Clean and maintenance：

1.  Always disconnect the power and leave to cool before cleaning.

2.  Clean the outside with a damp cloth. Do not use scouring powder, nonscratch cream cleanser or any other cleaning products as they may discolour your kettle.

- Please regularly clean out incrustation to ensure the normal working of the kettle. Do not scratch with metal material. Wash with diluted lemon extract.

- Never immerse the kettle, power base, cord, plug in water.

5.  Please cleaning this unit and put it into gift box after long time not to use, avoid this product are humid, please keep it in a dry place.

Trouble shooting：

1.  Power off and the indicator goes off before water boiled.

Possible causes:

a. Too much incrustation. The kettle resumes normal working after incrustation removed from the kettle.

b. Water blow the minimum water level mark. Add more water accordingly. Reset and the kettle will resume normal working.

c. Incorrect operation.

2.  Cannot reset after the water is boiled off and the power is cut off. Pour in water and wait for 5 minutes .The kettle will automatically reset and begin to boil water.

3.  The kettle does not working.

Check the connection of power supply. If the anti-dry- heating protector does not reset after protective operation is finished, unplug the kettle and operate after the kettle cools down.

4.  Water has a smell of plastic.

Usually, there will be a smell of plastic only at the first time when you use this kettle. Please refer to "Precautions" for solution. If the problem remains, fill the kettle with water up to the maximum water level mark and 5g of soda, boil and discard wate.

5.  Overheating and lamp flashing will happen if you connect the power and press the SWITCH(off) button when waterless. Please refill the water into the kettle, the power supply will be automatically turn on, and the problem would be settled.

- Never attempt the kettle repair by yourself, please contact with the distributor or our service center for repairing.

Product specifications：

| Name | Cordless Automatic Electric Kettle |
| --- | --- |
| Model | ZDH125K |
| Minimum capacity | 500ml |
| Maximum capacity | 1600ml |
| Power input | 2200W |
| Power supply | 230V～ 50Hz |
| Dimensions （L×W×H） | 140mm×200mm×350mm |